# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply of Procurement of a firefighting and rescue vehicle and firefighting suit sets	p 1 /…

Publication reference: HUSRB/23R/11/058/5.1.1

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Firefighting and rescue vehicle – quantity 1 The vehicle is intended for transporting people and equipment under both road and off-road conditions, in accordance with the current legal regulations of the Republic of Serbia, the Rulebook on the Classification of Motor and Trailer Vehicles, and the technical requirements for vehicles in road traffic, as well as the Law on Traffic Safety. - Drivetrain 4 x 4 - Fuel: gasoline - Engine power: min 140kW - Engine cubic capacity: min 1900 cm3 - Safety: ABS, TCS, ESC, ESP, HAC/HDC, TPMS, front airbags for driver and co-driver - AC with dust filter - Hydraulic servo steering - Steering wheel adjustment - Rerar parking sensors - Cruise control - Central door lock - Electric window opening - Full size spare tyre |  |  |  |
|  | Customization for water tank, high pressure pump, installation, and equipment. Made from non-corrosive material, connected to the chassis - At least 2 side compartments and one rear compartment with full protection against external conditions and lockable. - Enough space to accommodate a water pump, water tank and other gear. - The water pump, water tank, installations and other gear should be fixed with carriers to prevent moving - Accessing the equipment should be in line with SRPS EN 1846-2 - At least two LED lamps on each top side to luminate the area around the vehicle, with one joint switch - LED lights inside the compartments - Blue LED signal lamps in the front mask - Blue LED signal lamps on the top rear side - On the cabin roof, an integrated light and sound signaling system, flashing beacons, an electronic "wailing" siren with a control panel, and a public address system with a built-in microphone on the front dashboard are required. Minimum power 100W, minimum length 120cm. - Water tank minimum 280 litres, hose for filling the water tank, valve for emptying, analogue level gauge - Foam tank – minimum 5 litres - High pressure pump – minimum 20 l/min at minimum 200 bar. Water tank minimum 300 l. - A gasoline engine with electric and manual start for operating the high-pressure pump. Minimum 9.6 kW. Provide electric energy from the battery of the pump. - A high-pressure reel, 390 g/m, equipped with a brake and a handle for winding the rubber hose. Miminum 60m length. |  |  |  |
| 2. | Firefighting suit sets – quantity 10 The suits contain a jacket, trousers, helmet, gloves, belt, boots (sizes should be negotiated with the Contracting Authority) Jacket – fire suspension technology (EN 469:2020): - Reflective striping R2 - Elevated collar (adjustable width) - Pocked for radio receiver - Place for name tag - Loop for lamp (triple safety, adjustable length and width) - Epaulette - Built in RFID - Inside pocket - Outside pockets - Sleeve cuff (adjustable with thermal bracelet) - Bodycam holder - Antenna and microphone holder - Elbow reinforcement - Quick zipper - HPX System |  |  |  |
|  | Trousers – fire suspension technology (EN 469:2020) - Reflective striping R1 - Pockets - Adjustable waist height - Cargo pockets outside (with knife pocket on the right side) - Carabiner loops (inside) - Knee reinforcement - Removable knee pads - Loop for hanging (inside, left, right, and back) - Quicklock harness (easy adjustment) - Ergonomic hanging tape - Name tag - Built in RFID |  |  |  |
|  | Helmet - Type A structural firefighting helmet must be fully tested and certified in accordance with EN443:2008 / EN16471:2014 / EN16473:201 / ISO16073 - The total weight of the helmet must not exceed 1280g with face visor, additional eye shield and ear/neck protection - The helmet should be equipped with a transparent or optionally gold-plated internal protective face visor. Certified in accordance with EN14458:2018 - The helmet should be equipped with an additional transparent or optionally tinted inner eye shield. Certified in accordance with EN14458:2018 - All visors must be rotatably mounted inside the helmet shell. - Visors must be anti-scratch coated on the outside and coated on the inside. - The helmet should be equipped with an aramid reinforced polyurethane shock absorber. - Adjustment for head size 50-67 cm will be possible with only one helmet shell - The helmet lamp should be explosion proof and removable without tools |  |  |  |
|  | Gloves - Gloves must be tested and certified according to EN 659:2003+A1:2008+AC:2009. - Protective gloves should be designed for normal firefighting operations, including search and rescue. - The back of the hand and the cuff will be made of high quality NOMEX III (or equivalent) with non-woven KEVLAR laminated on the back for heat and cut protection - The palm side will be made of silicon-carbon mineral coated NOMEX/KEVLAR (or equivalent)-dual fabric for excellent dexterity, high abrasion resistance and anti-slip on wet and smooth surfaces - Gloves should be equipped with a waterproof and breathable GORE-TEX (or equivalent) moisture barrier on the knit - Outer fabric color: dark blue - Gloves should be designed with PU impact protectors on the back of the hand - Available sizes: 6, 7, 8, 9, 10, 11, 12, 13 |  |  |  |
|  | Belt (DIN EN 358:2000 and DIN 14927:2005 - The fire belt consists of 85 mm wide black belt tape - The belt at one end must have a rectangular lock with two teeth, through which the other end of the strap is passed - on which there is a part of the length of the eyelets that are arranged in pairs. The eyelets are used to position the fire belt to the desired length - On the side of the fire belt there is a loop to which a 770 mm long safety rope is attached (including clutch and carabiner) for securing firefighters during interventions - The rope must be made of twisted polyester cable (rope) fi 12 mm with a multi-functional automatic closing carabiner that can be locked with a Twistiock lock or a coil lock. |  |  |  |
|  | Boots (EN 15090:2012 – F2A HI3 CI AN SRC, type 2, class code I, design “D”) - Fireproof leather boot, with membrane, boot without laces - Type examination certificate according to EU Regulation 2016/425 - The boots should be maintained with standard care products, which are suitable for leather shoes with a membrane - Reflective elements made of reflective material - The sole should have integrated steel protection against perforation. Steel toe cap. - The membrane must not be completely glued to the boot but installed in a suspended position. This allows for the best possible air circulation with every step - Slip resistant sole (SRC) - Color: black - Available sizes: EU36 – EU51 |  |  |  |
|  | Items 1. and 2. should be delivered with brochures, instructions, certificates. Item 1: Firefighting and rescue vehicle should have the wording “ВАТРОГАСЦИ“ on the hood of the vehicle. Letter hight – 10cm, letter line width 1.5cm. The contractor must provide service coverage within the territory of the Republic of Serbia for all installed parts and equipment. The contractor undertakes to deliver all the documentation necessary for the registration and homologation of the vehicle upon delivery. The contractor is obligated to provide regular annual servicing of the customization and chassis during the warranty period and to guarantee service (including the supply of spare parts) for the customization for a minimum of 10 years after the warranty expires. The built in water tank must have a fire type A and B certificate (DIN EN 2 – EU P3-1/01). The high-pressure pump and water tank must have a guaranty of minimum 12 months. |  |  |  |